UKRO National Competition

Lesson Plan – Internal space creation techniques

| Session No: |  |
| --- | --- |
| Duration: | 50mins ? |
| Title: | Practical cutting techniques |
| Session Aims: | To impart an understanding of basic tool operations on a vehicle, by practical demonstration. |
| Objectives / Learning Outcomes: | By the end of this presentation students will be able to: - Identification where reformation can be employed internally - Demonstrate basic centre console and cross ramming techniques - Understand the hazards associated with SRS activation when ramming from the centre console - Understand the limits and capabilities of hydraulic and e Tool technologies when employing ramming |
| Resources Required: | At least one vehicle, as large as possible. Hydraulic/e tool rams and spreaders, PPE |
| Additional Notes: | This session is completely practical. The time of the session can be increased / decreased at the beginning by revisiting stability, glass management, vehicle preparation, and setting up a tool dump. The vehicle should be stabilised for this session, to create a realistic scenario, and demonstrate how forces applied to the car will affect the stability. Stability should be revisited as appropriate. The term ‘ramming’ may also incorporate spreading. The state and type of the vehicle will dictate how many times students can practise the different techniques. This lesson plan is written so that as many techniques can be practised on one vehicle as possible. This session is for a car on all four wheels, or alternatively on its roof and side. |
| Assessment Method: | Summative assessment through questions, and practical demonstration throughout session. |
| Reference Material: | Fire & Rescue Service Manual Volume 2 (Incidents Involving rescue from road vehicles. |

| # Time | # Instructor Activity | ## Student Activity | # Assessment | # Resources |
| --- | --- | --- | --- | --- |
| 0 >1 min | Introduction to Instructor and session. Introduce yourself and the session you’re going to deliver. | Listen | # Questioning | None |
| 1 > 10 mins | Aim & Objectives of presentation Read out the aim and main objectives and make sure students understand the purpose of the session. End with health and safety brief. | Listen | # Questioning | None |
| 10 – 48 mins | Rams/spreaders This section of the session should be to allow students to practise with the cutters. Ensure the session includes: - Operator positioning - Anticipation of tool movement - Positioning of tool and suitable position points - Rotation of crews | Listen, respond to questions, Practical demonstration | # Questioning, practical demon-stration, peer review. | Vehicle, hydraulic/e tool equipment, full PPE. |
| 48 -50mins | Summary and recap Q& A students about best uses, common mistakes such as operator positioning etc. |  |  |  |